Removing the foot roller assembly

Remove the leg and foot assembly

Remove the two pins of the fixed leg assembly

Remove that 3PCS screws

Remove the protective cover and the baffle

Remove the cable tie

Remove the 4PCS screw

Remove the calf protective cover

Remove the 8PCS screws that lock the legs and feet

Remove the screws,  then remove the foot bottom cover

Remove the leg and foot fixing cover

Unzip it, separate the leg and foot

Remove the 4PCS screws

After removing the foot roller,it can be replaced

Removing the leg kneading assembly

Turn the calf to the bottom and unzip

Open the zipper, pull down the holster to expose the back cover, and then remove the screws

After removing the leg kneading assembly,please install new one

Remove the back cover and continue to remove the screws of the leg holder

Remove the leg holder,  then continue to remove screws of the leg kneading assembly